Ghosting Tests – TestBoard_ADC124:

Manual ghosting test:

- Method: Press each TOP sensor and look for ghosting on the adjacent sensor in Octo-7, which is spread across 4 MUXs. (only 1 full octo per MUX + partial octo-7)

| Array | MUX (resistor value) | Channel pressed | Channel tested |
| --- | --- | --- | --- |
| 1 | 1 470K) | PZT1_T (ch4) | PZT7_B (ch5) |
| 1 | 2 (1M) | PZT3_T (ch4) | PZT7_ L (ch5) |
| 1 | 3 (470K) | PZT5_T (ch4) | PZT7_C (ch5) |
| 1 | 4 (1M) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) |
| 2 | 1 (1M) | PZT1_T (ch4) | PZT7_B (ch5) |
| 2 | 2 (470K) | PZT3_T (ch4) | PZT7_ L (ch5) |
| 2 | 3 (1M) | PZT5_T (ch4) | PZT7_C (ch5) |
| 2 | 4 (470K) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) |

- Ghosting on the same sensor channels is harder to test – hard to distinguish between ghosting and physical coupling with a manual press.

- Tested with SPI = 16Mhz.

- Tested on different sensor settling time and vmid settling time (when vmid = on)

- With vmid ON, tested with ADC scan after switching to vmid, and without ADC scan.

- Measured relative ghosting and peak-to-peak amplitude of the major signals. Pressure was applied manually (not a systematically accurate method) and by observation, not an accurate measurement.

Results with Vmid OFF:

Sensor settling time = 5us. ADC scan.

| Array | MUX | Pressed | Tested | Results | Sig p-p |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 470K) | PZT1_T (ch4) | PZT7_B (ch5) | Very strong ghost (90%) | 30mv |
| 1 | 2 (1M) | PZT3_T (ch4) | PZT7_ L (ch5) | Very strong ghost (90%) | 80mv |
| 1 | 3 (470K) | PZT5_T (ch4) | PZT7_C (ch5) | Very strong ghost (90%) | 60mV |
| 1 | 4 (1M) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) | Very strong ghost (90%). Ghost on second channel. | 80mv |
| 2 | 1 (1M) | PZT1_T (ch4) | PZT7_B (ch5) | Strong ghost (50%) | 180mv |
| 2 | 2 (470K) | PZT3_T (ch4) | PZT7_ L (ch5) | Ghost (30%) | 150mv |
| 2 | 3 (1M) | PZT5_T (ch4) | PZT7_C (ch5) | Ghost (30%) | 250mv |
| 2 | 4 (470K) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) | Ghost (30%). Hardly any ghost on second channel. | 100mv |

Results with Vmid ON:

Sensor settling time = 5us. ADC scan.

Vmid settling time = 0 (+ ADC scan time after vmid switching).).

Sample interval 5us. SPS 4140Hz

| Array | MUX | Pressed | Tested | Results | Sig p-p |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 (470K) | PZT1_T (ch4) | PZT7_B (ch5) | No ghost | 30mv |
| 1 | 2 (1M) | PZT3_T (ch4) | PZT7_ L (ch5) | No ghost | 60mv |
| 1 | 3 (470K) | PZT5_T (ch4) | PZT7_C (ch5) | No ghost | 40mV |
| 1 | 4 (1M) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) | No ghost | 40mv |
| 2 | 1 (1M) | PZT1_T (ch4) | PZT7_B (ch5) | No ghost | 180mv |
| 2 | 2 (470K) | PZT3_T (ch4) | PZT7_ L (ch5) | No ghost | 150mv |
| 2 | 3 (1M) | PZT5_T (ch4) | PZT7_C (ch5) | Very small ghost (<5%) | 250mv |
| 2 | 4 (470K) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) | No ghost | 100mv |

Sensor settling time = 5us. MUX scan.

Vmid settling time = 0 (+ ADC scan time after vmid switching).

Sample interval 4us. SPS 5000Hz

| Array | MUX | Pressed | Tested | Results | Sig p-p |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 (470K) | PZT1_T (ch4) | PZT7_B (ch5) | No ghost | 30mv |
| 1 | 2 (1M) | PZT3_T (ch4) | PZT7_ L (ch5) | No ghost | 30mv |
| 1 | 3 (470K) | PZT5_T (ch4) | PZT7_C (ch5) | Almost no ghost (<1%, noise pattern changes) | 30mV |
| 1 | 4 (1M) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) | No ghost | 35mv |
| 2 | 1 (1M) | PZT1_T (ch4) | PZT7_B (ch5) | No ghost | 150mv |
| 2 | 2 (470K) | PZT3_T (ch4) | PZT7_ L (ch5) | No ghost | 60mv |
| 2 | 3 (1M) | PZT5_T (ch4) | PZT7_C (ch5) | Very small ghost (<5%) | 80mv |
| 2 | 4 (470K) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) | No ghost | 50mv |

Sensor settling time = 1us. ADC scan.

Vmid settling time = 0 (+ ADC scan time after vmid switching).

Sample interval 4us. SPS 5375Hz

| Array | MUX | Pressed | Tested | Results | Sig p-p |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 (470K) | PZT1_T (ch4) | PZT7_B (ch5) | No ghost | 60mv |
| 1 | 2 (1M) | PZT3_T (ch4) | PZT7_ L (ch5) | No ghost | 60mv |
| 1 | 3 (470K) | PZT5_T (ch4) | PZT7_C (ch5) | No ghost | 40mV |
| 1 | 4 (1M) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) | No ghost | 60mv |
| 2 | 1 (1M) | PZT1_T (ch4) | PZT7_B (ch5) | No ghost | 200mv |
| 2 | 2 (470K) | PZT3_T (ch4) | PZT7_ L (ch5) | No ghost | 150mv |
| 2 | 3 (1M) | PZT5_T (ch4) | PZT7_C (ch5) | Very small ghost (<1%) | 250mv |
| 2 | 4 (470K) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) | Almost no ghost (<1%, noise pattern changes) | 200mv |

Sensor settling time = 1us. ADC scan.

Vmid settling time = 0 (NO ADC scan after vmid switching).

Sample interval 2us. SPS 9320Hz

| Array | MUX | Pressed | Tested | Results | Sig p-p |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 (470K) | PZT1_T (ch4) | PZT7_B (ch5) | No ghost | 40mv |
| 1 | 2 (1M) | PZT3_T (ch4) | PZT7_ L (ch5) | No ghost | 40mv |
| 1 | 3 (470K) | PZT5_T (ch4) | PZT7_C (ch5) | No ghost | 40mV |
| 1 | 4 (1M) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) | No ghost | 50mv |
| 2 | 1 (1M) | PZT1_T (ch4) | PZT7_B (ch5) | Very small ghost (<2%) | 100mv |
| 2 | 2 (470K) | PZT3_T (ch4) | PZT7_ L (ch5) | Very small ghost (<1%) | 40mv |
| 2 | 3 (1M) | PZT5_T (ch4) | PZT7_C (ch5) | Very small ghost (<1%) | 50mv |
| 2 | 4 (470K) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) | Almost no ghost (<1%, noise pattern changes) | 40mv |

Sensor settling time = 1us. ADC scan.

Vmid settling time = 5 (NO ADC scan after vmid switching).

Sample interval 4us. SPS 5620Hz

| Array | MUX | Pressed | Tested | Results | Sig p-p |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 (470K) | PZT1_T (ch4) | PZT7_B (ch5) | No ghost | 50mv |
| 1 | 2 (1M) | PZT3_T (ch4) | PZT7_ L (ch5) | No ghost | 50mv |
| 1 | 3 (470K) | PZT5_T (ch4) | PZT7_C (ch5) | No ghost | 40mV |
| 1 | 4 (1M) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) | No ghost | 70mv |
| 2 | 1 (1M) | PZT1_T (ch4) | PZT7_B (ch5) | No ghost | 200mv |
| 2 | 2 (470K) | PZT3_T (ch4) | PZT7_ L (ch5) | No ghost | 150mv |
| 2 | 3 (1M) | PZT5_T (ch4) | PZT7_C (ch5) | Very small ghost (<1%) | 250mv |
| 2 | 4 (470K) | PZT6_T (ch4) | PZT7_R (ch5), T(ch6) | Almost no ghost (<1%, noise pattern changes) | 200mv |

Tests above were for blocking SPI engine.

For LPSPI engine these are the sampling rates:

| Scan Mode | Channel Settle [us] | Vmid Settle [us] | ADC scan after vmid | Blocking: Channel SPS [Hz] | Blocking: Interval [us] | LPSPI: Channel SPS [Hz] | LPSPI: Interval [us] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ADC | 5 | (no vmid) | (no vmid) |  |  | 6736 | 3 |
| ADC | 5 | 0 | yes | 4140 | 5 |  |  |
| MUX | 5 | 0 | yes | 5000 | 4 |  |  |
| ADC | 1 | 0 | yes | 5375 | 4 |  |  |
| ADC | 1 | 0 | no | 9320 | 2 | 10000 | 2 |
| ADC | 1 | 5 | no | 5620 | 4 | 5880 | 3 |

Comparing TestBoard_7953 and TestBoard_ADC124 Signals:

Using sensor strip: v12b 8/5/26

TestBoard_7953:

Vmid on, 20Mhz SPI, Ref:2.5V, Sampling rate (GUI): 6750 Hz

TestBoard_ADC124:

Vmid on, Settle: 1us on channel, 5us on vmid, ADC mode, Sampling rate (GUI): 5880 Hz

Testing only array2

TestBoard PCB1.7:

Vmid on, Sampling rate (GUI): 680Hz

| Sensor | 7953_array1 [mv] | 7953_array2 [mv] | ADC124_array2 [mv] | PCB1.7 [mv] |
| --- | --- | --- | --- | --- |
| PZT1_T | 200 | 200 | 200 | 650 |
| PZT1_C | 150 | 150 | 200 | 600 |
| PZT3_T | 250 | 150 | 150 | 500 |
| PZT3_C | 300 | 250 | 250 | 400 |
| PZT5_T | 250 | 250 | 250 | 1000 |
| PZT5_C | 250 | 200 | 300 | 1000 |
| PZT6_T | 200 | 150 | 150 |  |
| PZT6_C | 200 | 200 | 200 |  |
| PZT7_T | 250 | 250 | 200 |  |
| PZT7_C | 150 | 250 | 250 |  |