Ghosting Tests – TestBoard_7953:

Manual ghosting test:

- Method: Press on each TOP sensor and look for ghosting on the adjacent sensor in the Mux -next sensor  BOTTOM and LEFT

| Array | ADC (resistor value) | Channel pressed | Channel tested |
| --- | --- | --- | --- |
| 1 | 1 (1M) | PZT6_T (ch4) | PZT7_B (ch5), L(ch6) |
| 1 | 2 (470K) | PZT1_T (ch4) | PZT3_B (ch5), L(ch6) |
| 1 | 2 (470K) | PZT3_T(ch_9) | PZT5_B(ch10), L(ch11) |
| 2 | 1 (470K) | PZT6_T (ch4) | PZT7_B (ch5), L(ch6) |
| 2 | 2 (249K) | PZT1_T (ch4) | PZT3_B (ch5), L(ch6) |
| 2 | 2 (249K) | PZT3_T(ch_9) | PZT5_B(ch10), L(ch11) |

- Ghosting on the same sensor channels is harder to test – hard to distinguish between ghosting and physical coupling with a manual press.

- Tested for different SPI speeds – 5MHz, 10MHz, 20MHz

- No ghosting noticed on prior channels in the ADC/MUX.

- No ghosting noticed between different ADC/MUXs.

Results with Vmid OFF:

SPI = 5Mhz

| Array | ADC | Channel pressed | Channel tested | Results |
| --- | --- | --- | --- | --- |
| 1 | 1 (1M) | PZT6_T (ch4) | PZT7_B (ch5) | Large ghosting (about 30%). Ghosting follows to the next channel (7_L) |
| 1 | 2 (470K) | PZT1_T (ch4) | PZT3_B (ch5) | Some ghosting (less than 20%). Ghosting doesn’t follow the next channel(3_L) |
| 1 | 2 (470K) | PZT3_T(ch_9) | PZT5_B(ch10) | Some ghosting (about 20%). Small ghosting follows the next channel(5_L) |
| 2 | 1 (470K) | PZT6_T (ch4) | PZT7_B (ch5) | Some ghosting. Small ghosting follows the next channel(5_L) |
| 2 | 2 (249K) | PZT1_T (ch4) | PZT3_B (ch5) | VERY small ghosting. Ghosting doesn’t follow the next channel(3_L) |
| 2 | 2 (249K) | PZT3_T(ch_9) | PZT5_B(ch10) | VERY small ghosting. Ghosting doesn’t follow the next channel(5_L) |

SPI = 10Mhz

| Array | ADC | Channel pressed | Channel tested | Results |
| --- | --- | --- | --- | --- |
| 1 | 1 (1M) | PZT6_T (ch4) | PZT7_B (ch5) | Strong ghosting (almost 50%). Also follows to the next channel (7_L) |
| 1 | 2 (470K) | PZT1_T (ch4) | PZT3_B (ch5) | Large ghosting (about 40%). Also follows to the next channel (3_L) |
| 1 | 2 (470K) | PZT3_T(ch_9) | PZT5_B(ch10) | Large ghosting (about 30%). Also follows to the next channel (5_L) |
| 2 | 1 (470K) | PZT6_T (ch4) | PZT7_B (ch5) | Large ghosting (about 40%). Also follows to the next channel (7_L) |
| 2 | 2 (249K) | PZT1_T (ch4) | PZT3_B (ch5) | Small ghosting but larger than 5Mhz. Ghosting hardly follows the next channel(3_L) |
| 2 | 2 (249K) | PZT3_T(ch_9) | PZT5_B(ch10) | Small ghosting but larger than 5Mhz. Ghosting hardly follows the next channel(5_L) |

SPI = 20Mhz

| Array | ADC | Channel pressed | Channel tested | Results |
| --- | --- | --- | --- | --- |
| 1 | 1 (1M) | PZT6_T (ch4) | PZT7_B (ch5) | Strong ghosting (more than 50%). Also follows to the next channel (7_L) |
| 1 | 2 (470K) | PZT1_T (ch4) | PZT3_B (ch5) | Large ghosting (about 40%). Also follows to the next channel (3_L) |
| 1 | 2 (470K) | PZT3_T(ch_9) | PZT5_B(ch10) | Strong ghosting (more than 50%). Also follows to the next channel (5_L) |
| 2 | 1 (470K) | PZT6_T (ch4) | PZT7_B (ch5) | Strong ghosting (more than 50%). Also follows to the next channel (7_L) |
| 2 | 2 (249K) | PZT1_T (ch4) | PZT3_B (ch5) | Larger than 10Mhz but smaller than 470K/1M tests. Very small ghosting follows the next channel(3_L) |
| 2 | 2 (249K) | PZT3_T(ch_9) | PZT5_B(ch10) | Larger than 10Mhz but smaller than prior tests. Very small ghosting follows the next channel(5_L) |

Results with Vmid ON:

SPI = 20Mhz

| Array | ADC | Channel pressed | Channel tested | Ghosting Results | Signal P-to-P |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 (1M) | PZT6_T (ch4) | PZT7_B (ch5) | Very small ghosting (less than 20%). No ghosting following the next channel. | 130mv |
| 1 | 2 (470K) | PZT1_T (ch4) | PZT3_B (ch5) | No ghosting observed. | 100mv |
| 1 | 2 (470K) | PZT3_T(ch_9) | PZT5_B(ch10) | No ghosting observed. | 150mv |
| 2 | 1 (470K) | PZT6_T (ch4) | PZT7_B (ch5) | Almost no ghosting observed (about 5%) |  |
| 2 | 2 (249K) | PZT1_T (ch4) | PZT3_B (ch5) | No ghosting observed. |  |
| 2 | 2 (249K) | PZT3_T(ch_9) | PZT5_B(ch10) | No ghosting observed. |  |